# General

Version 2023 okt

| City: | Tiel |
| --- | --- |
| Theatre: | Schouwburg & Filmtheater Agnietenhof |
| Venue: | Kontakt Mediapartners Zaal, grote zaal |
| Adress: | St.Agnietenstraat 2 |
| Postal code addres: | 4001 NB |
| Postal adress: | Postbus 560 |
| Postal code adres : | 4000 AN Tiel |

| Administration: | 0344 673 515 |
| --- | --- |
| Tickets: | 0344 673 500 |
| Technical Team: | 0344 673 510 |

| Email administration: | info@cultuurbedrijftiel.nl |
| --- | --- |
| Email technical team: | techniek@cultuurbedrijftiel.nl |
| Website: | www.agnietenhof.nl |

| Director: | Nico Baars |
| --- | --- |
| Publicity: | Floor Marcusse |
| Technical manager | Bas de Beer |
| Stage manager: | Steven Ram |
| Lunch time: | i.o., minimaal 45 min. |
| End of day shift: | 17.00 uur |
| Start evening shift: | 19.00 uur |
| Start show: | 20.00 uur |
| Doors open theatre between 17:00 en 19:00: |  |
| Maximum number of seats: | 796 |
| Seats on orchestra pit lift: | 34 |
| Seats excl. balcony: | 472 (incl.seats on orchestra pit lift) |
| Seats reserved of FOH: | 4 to 10 (depending on size controls) |
| Seats on balcony: | 324 |

## Dressing rooms

There are ten dressing rooms available, shared with the small venue. One is next to the main stage, the other nine are one floor up, reachable by lift or stairs. Two of these dressing rooms have their own shower and bathroom. The other seven have shared one-person showers and bathrooms.

The theatre has a washing machine and dryer available.

## Loading, unloading and parking facilities

One lorry can park inside, cars can park in the adjacent parkinggarage (max. height 2.1m). A parking ticket needs to be bought, the theatre does not have permits. Address loading dock: crossroads Eerste Bleekveldstraat – Koninginnestraat.

## Dimensions

### Stage opening

### Main opening

Widt:	12	m

Height:	7	m

### Portal bridge height (Pros Arch)

Minimally:	0,00	m

Maximally:	7,15	m

### Lighting towers opening width (Pros Arch)

Minimally:                                                           	10  m

Maximally:                                                          	14  m

## Stage

### Depth from point 0

Rear wall auditorium:	-2037	cm

Balcony edge:	-1552	cm

Front orchestra pit (m):	-592	cm

Front stage:	-317	cm

Rear side main opening:	-191	cm

Curtain:	-153	cm

Front lighting towers opening:	-86	cm

Rear bridge:	986	cm

Rear wall stage:	1065	cm

### Width from centre line

Side bridge left:	994	cm

Side wall tower left:	1298	cm

Side wall left:	1900	cm

Side bridge right:	1005	cm

Side wall tower right:	1304	cm

### Height

Side and rear bridges	780	cm

Underside fly-bar loft:	1960	cm

Stage height:	-108	cm

Floor

Wooden Keroewing floor, max. load capacity 500 kg / m2

# Inrichting

## Draperies

### Main curtain

| Colour | Red |
| --- | --- |
| Controls | Manually on stage left |
| Beweging | Grieks/Wagner |

### Legs

| Number | 10 |  |  |  |
| --- | --- | --- | --- | --- |
| Height | 9 m |  |  |  |
| Width | 4,8 m |  |  |  |
| Colour | Night blue (Navy) |  |  |  |
| Material | Velvet |  |  |  |

### Backdrop (ussenfond) halfway (5 legs)

| Number | 5 |  |
| --- | --- | --- |
| Height | 9 m |  |
| Colour | Night Blue (Navy) |  |
| Material | Velvet |  |

### Borders

| Number | 6 |  |
| --- | --- | --- |
| Height | 4 m |  |
| Width | 18.60 m |  |
| Colour | Night Blue (Navy) |  |
| Material | Velours |  |

### Backdrop (fond) (5 legs)

| Number | 5 |  |
| --- | --- | --- |
| Height | 9 m |  |
| Colour | Night Blue (Navy) |  |
| Material | Velvet |  |

### Cyclorama (horizon)

| Number | 1 |  |
| --- | --- | --- |
| Height | 10 m |  |
| Width | 18 m |  |
| Colour | Off White |  |

### Scrim / Gauze

| Number: | 1 |  | 1 |  |
| --- | --- | --- | --- | --- |
| Height | 9,5 m |  | 9,5 m |  |
| Width | 14 m |  | 14 m |  |
| Colour | Black |  | Grey |  |

# Fly system

## Technical data

### General

Constructer	Trekwerk

System	TNM

Control location	right

evt. left

### Fly bars 300 kg, automated

Number	42

Length	18,60	m

Circumference	48	mm

Type	  Single pipe

Max load	300	kg

Max load underneath cable	150	kg

Max load between cables	100	kg

Number of cables per fly bar	7

Distance between cables	2,70	m

Maximum speed	1,7	m/s

Heigth fly bar

Minimally	0,25	m

Maximally	18,80	m

### Fly bars 300 kg (51 & 52), automated

### Number	2	Stuks

### Length	18,60	M

### Circumference	48	Mm

### Type	     ladder truss

### Max load	300	Kg

### Max load underneath cable	250	Kg

### Max load between cables	250	kg

### Number of cables per fly	7	stuks

### Distance between cables	2,70	m

### Maximum speed	1,7	m/s

### Heigth

### Minimally	0,25	m

### Maximally	18,20	m

### Side fly bars 300 kg, automated

Number	2	Stuks

Length	9,30	m

Circumference	48	mm

Type	Ladder truss

Max load	300	kg

Max load underneath cable	150	kg

Max load between cables	100	kg

Number of cables per fly bar	3	stuks

Distance between cables	Var.	m

Maximum speed	0,8	m/s

Height fly bar

Minimally	0,25	m

Max	18,20	m

Distance between side fly bar and sidebars 	20	cm

Distance between side fly bar and side bridge	20	cm

Traversesystem

3 x 3 Traverses

WLL 1000 kg

3 in front off fly-bar 51

3 in between fly-bar 51 and 52

3 behind fly-bar 52

Underside beam	21,60 m

### Orchestra pit lift

Length	         10	M

Width	2,63	M

Dividable	        no

Vergrendelbaar op	all height

Type	Spindellift

## Instruments

Available on request

Steinway  Vleugel C227	440 Hz

Yamaha Vleugel  C3		440 Hz

Bechstein Piano 	440 Hz

# Light

## Lighting desks

| Lightcomputer | ETC ION met 20ch wing |  |
| --- | --- | --- |
| Also available | ETC Element (kleine zaal) Compulite Spark Top |  |
| Remote focussing | Yes |  |
| Moveble | Yes |  |
| Hall light on lightdesk | Yes |  |

## Dimmers, connectors and power

### Fixed Dimmers

| Brand/type | ETC | Sensor 3 |
| --- | --- | --- |
| Location | Kelder |  |
| Power | 3 | Kw |
| Total circuits | 126 | Circuits |

All dimmer circuits are switchable

### Mobile dimmerracks

| Number of racks | 2 | Racks |
| --- | --- | --- |
| Brand/type | ETC Alex |  |
| Power | 2,5 | Kw |
| Total circuits per dimmer | 24 | Kringen |

### Connectors

| Type connectors | Schuko |
| --- | --- |

### Power 3-phase

Left front:     2 x CEE 125 A

Right front:   1 x  CEE 125 A

Stage left     1x CEE 32 A

Stage right   2 x  CEE 32 A

Stage right   1 x  CEE 16 A

## Light

See our lightingplan

### Horizon

In Fly-bar			8* ETC ColorSource Cyc

Floor (optional)	              		7* ETC ColorSource Cyc

Programmable through presets or by lighting concole

## Followspots

| Robert Juliat Korrigan 5/9 gr | 1 | Stuks |
| --- | --- | --- |
| For Main Stage |  | 1,2 HMI |
| Robert Juliat Buxie 10,5/22,5 | 1 | Stuks |
| For Small stage |  | 0,575 MSR |

## Mobile Spots

Mobile spots are available on request. Mobile spots can be used in Main Stage and Small Stage.

| Altman Zoom 1 Kw | 8 |  |
| --- | --- | --- |
| Sixbar Par 64 1 Kw MFL | 4 |  |
| PC 2 Kw | 8 |  |
| PC 1 Kw | 12 |  |
| Fourbar Par 64 1 KW MFL | 6 |  |
| Par 64 1 Kw los | 8 |  |
| ETC Source Four multi par 575 W | 8 |  |

ETC Color source LED par	          24

Clay Packy Alpha Spot 700	          8       (available on request)

Moving light available on request

## Theatre lift

| Brand/type (2 x, for MS and KZ) |  | Alp PHCI 940 |
| --- | --- | --- |
| Height |  | 8 m |
| Power |  | 230V |

## Audiovisual equipment

| Multiple beamers and screens available on request. |  |  |
| --- | --- | --- |

# Sound

## Soundsystem

### PA

| Sub | Meyer Sound 700 HP |
| --- | --- |
| Number | 2 |
| FOH | Meyer Sound UPQ-1P |
| Number | 2 |
| Balcony L+R | Meyer Sound Mina |
| Number | 2x2 |
| Frontfill L+R | Meyer Sound UPJunior |
| Aantal | 2 |
| Cluster | Meyer Sound UPJ-1P |

Soundsystem is controlled by Galileo DSP.

Monitoring

10 x Sound Projects X-act

2  x  Sound Projects X-Tender

(Available on request)

### miscellaneous

| Siap Controll System | Acoustic Enhancement system |  |  |
| --- | --- | --- | --- |
| Auditief geh install | Audio-technica 580.00MHz |  |  |
| Intercom |  |  |  |
| Riedel | Wireless, 7 wireless beltpacks |  |  |

Mixing consoles

Yamaha QL1

Yamaha TF3

(Available on request)

Microphones / stands / cables

| Microphones on request |  |  |
| --- | --- | --- |
| Wireless microphones on request |  |  |